PROJECT :

Network Design for Faculty of Computing Block N28c

TASK 5: IP ADDRESSING SCHEME

SECR1213 - NETWORK COMMUNICATIONS

SEMESTER I, SESSION 2024/2025

SECTION 02

Lecturer : Dr. ISMAIL FAUZI BIN ISNIN

| NAME | Matric.No |
| --- | --- |
| TAN ZHI MING | A23CS0189 |
| NG YU HIN | A23CS0148 |
| LEE YIN SHEN | A23CS0236 |
| MUHAMMAD NAIM BIN ABDULLAH | A23CS0134 |

Contents

5.1 Get the Network Address from your lecturer.	2

5.2 Divide it in the best possible way for your network.	3

5.2.1 Network distribution for different work area	3

5.2.2 Network distribution for workstation in different work areas.	6

Ground Floor :	6

First Floor:	12

Meeting Minute #1	16

### 5.1 Get the Network Address from your lecturer.

| Group | Network IP address |
| --- | --- |
| 1 | 10.16.0.0/12 |
| 2 | 10.32.0.0/12 |
| 3 | 10.48.0.0/12 |
| 4 | 10.64.0.0/12 |
| 5 | 10.80.0.0/12 |
| 6 | 10.96.0.0/12 |
| 7 | 10.112.0.0/12 |
| 8 | 10.128.0.0/12 |
| 9 | 10.144.0.0/12 |
| 10 | 10.176.0.0/12 |

Figure 5.1.1

Figure 5.1.1 shows the list of network addresses of each group. The network address of our group is 10.32.0.0/12. We assume that our subnet mask is /24 or 255.255.255.0 .

Network Address details:

| IP address (decimal) | 10.32.0.0 |
| --- | --- |
| IP address (binary) | 00001010 00100000 00000000 00000000 |
| Subnet Mask (decimal) | 255.255.255.0 |
| Subnet Mask (binary) | 11111111 11111111 11111111 00000000 |

| Network Address | 10.32.0.0 |
| --- | --- |
| Broadcast Address | 10.32.255.255 |
| Range of useable address | 10.32.0.1 - 10.32.255.254 |

### 5.2 Divide it in the best possible way for your network.

### 5.2.1 Network distribution for different work area

| subnet | Network portion | Subnet bit | Host portion |  | IP address |
| --- | --- | --- | --- | --- | --- |
| 0 | 00001010 0010 | 0000 00000000 | 00000000 | Network address | 10.32.0.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.0.255 |
| 1 | 00001010 0010 | 0000 00000001 | 00000000 | Network address | 10.32.1.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.1.255 |
| 2 | 00001010 0010 | 0000 00000010 | 00000000 | Network address | 10.32.2.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.2.255 |
| 3 | 00001010 0010 | 0000 00000011 | 00000000 | Network address | 10.32.3.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.3.255 |
| 4 | 00001010 0010 | 0000 00000100 | 00000000 | Network address | 10.32.4.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.4.255 |
| 5 | 00001010 0010 | 0000 00000101 | 00000000 | Network address | 10.32.5.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.5.255 |
| 6 | 00001010 0010 | 0000 00000110 | 00000000 | Network address | 10.32.6.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.6.255 |
| 7 | 00001010 0010 | 0000 00000111 | 00000000 | Network address | 10.32.7.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.7.255 |
| 8 | 00001010 0010 | 0000 00001000 | 00000000 | Network address | 10.32.8.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.8.255 |
| 9 | 00001010 0010 | 0000 00001001 | 00000000 | Network address | 10.32.9.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.9.255 |
| 10 | 00001010 0010 | 0000 000010010 | 00000000 | Network address | 10.32.10.0 |
|  |  |  | 11111111 | Broadcast address | 10.32.10.255 |

| Subnet | Work area | Network address | Broadcast address | Range of usable address | Subnet mask address |
| --- | --- | --- | --- | --- | --- |
| 0 | Database room & Server room | 10.32.0.0 | 10.32.0.255 | 10.32.0.1 - 10.32.0.254 | 255.255.255.0/24 |
| Ground Floor |  |  |  |  |  |
| 1 | General lab 1 | 10.32.1.0 | 10.32.1.255 | 10.32.1.1 - 10.32.1.254 | 255.255.255.0/24 |
| 2 | General lab 2 | 10.32.2.0 | 10.32.2.255 | 10.32.2.1 - 10.32.2.254 | 255.255.255.0/24 |
| 3 | Hybrid classroom | 10.32.3.0 | 10.32.3.255 | 10.32.3.1 - 10.32.3.254 | 255.255.255.0/24 |
| 4 | Voice conference room | 10.32.4.0 | 10.32.4.255 | 10.32.4.1 - 10.32.4.254 | 255.255.255.0/24 |
| 5 | Student lounge | 10.32.5.0 | 10.32.5.255 | 10.32.5.1 - 10.32.5.254 | 255.255.255.0/24 |
| First Floor |  |  |  |  |  |
| 6 | NOC | 10.32.6.0 | 10.32.6.255 | 10.32.6.1 - 10.32.6.254 | 255.255.255.0/24 |
| 7 | ESL | 10.32.7.0 | 10.32.7.255 | 10.32.7.1 - 10.32.7.254 | 255.255.255.0/24 |
| 8 | CNL | 10.32.8.0 | 10.32.8.255 | 10.32.8.1 - 10.32.8.254 | 255.255.255.0/24 |
| 9 | MTR | 10.32.9.0 | 10.32.9.255 | 10.32.9.1 - 10.32.9.254 | 255.255.255.0/24 |
| 10 | MR | 10.32.10.0 | 10.32.10.255 | 10.32.10.1 - 10.32.10.254 | 255.255.255.0/24 |

### 5.2.2 Network distribution for workstation in different work areas.

### Ground Floor :

General Lab 1 :

| Device | IP address |
| --- | --- |
| PC1 | 10.32.1.1 |
| PC2 | 10.32.1.2 |
| PC3 | 10.32.1.3 |
| PC4 | 10.32.1.4 |
| PC5 | 10.32.1.5 |
| PC6 | 10.32.1.6 |
| PC7 | 10.32.1.7 |
| PC8 | 10.32.1.8 |
| PC9 | 10.32.1.9 |
| PC10 | 10.32.1.10 |
| PC11 | 10.32.1.11 |
| PC12 | 10.32.1.12 |
| PC13 | 10.32.1.13 |
| PC14 | 10.32.1.14 |
| PC15 | 10.32.1.15 |
| PC16 | 10.32.1.16 |
| PC17 | 10.32.1.17 |
| PC18 | 10.32.1.18 |
| PC19 | 10.32.1.19 |
| PC20 | 10.32.1.20 |
| PC21 | 10.32.1.21 |
| PC22 | 10.32.1.22 |
| PC23 | 10.32.1.23 |
| PC24 | 10.32.1.24 |
| PC25 | 10.32.1.25 |
| PC26 | 10.32.1.26 |
| PC27 | 10.32.1.27 |
| PC28 | 10.32.1.28 |
| PC29 | 10.32.1.29 |
| PC30 | 10.32.1.30 |
| PC31 | 10.32.1.31 |
| ACCESS POINT 1 | 10.32.1.32 |
| Printer | 10.32.1.33 |
| Photostat Machine | 10.32.1.34 |
| BigScreenTV | 10.32.1.35 |

General Lab 2 :

| Device | IP address |
| --- | --- |
| PC1 | 10.32.2.1 |
| PC2 | 10.32.2.2 |
| PC3 | 10.32.2.3 |
| PC4 | 10.32.2.4 |
| PC5 | 10.32.2.5 |
| PC6 | 10.32.2.6 |
| PC7 | 10.32.2.7 |
| PC8 | 10.32.2.8 |
| PC9 | 10.32.2.9 |
| PC10 | 10.32.2.10 |
| PC11 | 10.32.2.11 |
| PC12 | 10.32.2.12 |
| PC13 | 10.32.2.13 |
| PC14 | 10.32.2.14 |
| PC15 | 10.32.2.15 |
| PC16 | 10.32.2.16 |
| PC17 | 10.32.2.17 |
| PC18 | 10.32.2.18 |
| PC19 | 10.32.2.19 |
| PC20 | 10.32.2.20 |
| PC21 | 10.32.2.21 |
| PC22 | 10.32.2.22 |
| PC23 | 10.32.2.23 |
| PC24 | 10.32.2.24 |
| PC25 | 10.32.2.25 |
| PC26 | 10.32.2.26 |
| PC27 | 10.32.2.27 |
| PC28 | 10.32.2.28 |
| PC29 | 10.32.2.29 |
| PC30 | 10.32.2.30 |
| PC31 | 10.32.2.31 |
| ACCESS POINT 1 | 10.32.2.32 |
| Printer | 10.32.2.33 |
| Photostat Machine | 10.32.2.34 |
| BigScreenTV | 10.32.2.35 |

Hybrid Classroom :

| Device | IP address |
| --- | --- |
| PC1 | 10.32.3.1 |
| PC2 | 10.32.3.2 |
| PC3 | 10.32.3.3 |
| PC4 | 10.32.3.4 |
| PC5 | 10.32.3.5 |
| PC6 | 10.32.3.6 |
| PC7 | 10.32.3.7 |
| PC8 | 10.32.3.8 |
| PC9 | 10.32.3.9 |
| PC10 | 10.32.3.10 |
| PC11 | 10.32.3.11 |
| PC12 | 10.32.3.12 |
| PC13 | 10.32.3.13 |
| PC14 | 10.32.3.14 |
| PC15 | 10.32.3.15 |
| PC16 | 10.32.3.16 |
| PC17 | 10.32.3.17 |
| PC18 | 10.32.3.18 |
| PC19 | 10.32.3.19 |
| PC20 | 10.32.3.20 |
| PC21 | 10.32.3.21 |
| PC22 | 10.32.3.22 |
| PC23 | 10.32.3.23 |
| PC24 | 10.32.3.24 |
| PC25 | 10.32.3.25 |
| PC26 | 10.32.3.26 |
| PC27 | 10.32.3.27 |
| PC28 | 10.32.3.28 |
| PC29 | 10.32.3.29 |
| PC30 | 10.32.3.30 |
| PC31 | 10.32.3.31 |
| PC32 | 10.32.3.32 |
| PC33 | 10.32.3.33 |
| PC34 | 10.32.3.34 |
| PC35 | 10.32.3.35 |
| PC36 | 10.32.3.36 |
| PC37 | 10.32.3.37 |
| PC38 | 10.32.3.38 |
| PC39 | 10.32.3.39 |
| PC40 | 10.32.3.40 |
| PC41 | 10.32.3.41 |
| Access Point 1 | 10.32.3.42 |
| Projector 1 | 10.32.3.43 |
| Projector 2 | 10.32.3.44 |
| BigScreenTV | 10.32.3.45 |

Voice conference room :

| Device | IP address |
| --- | --- |
| PC1 | 10.32.4.1 |
| Projector 1 | 10.32.4.2 |
| Photostat Machine | 10.32.4.3 |
| BigSceenTV | 10.32.4.4 |
| ACCESS POINT 1 | 10.32.4.5 |

Student lounge:

| Device | IP address |
| --- | --- |
| BigScreenTV | 10.32.5.1 |

### First Floor:

NOC :

| Device | IP address |
| --- | --- |
| PC1 | 10.32.6.1 |
| PC2 | 10.32.6.2 |
| PC3 | 10.32.6.3 |
| PC4 | 10.32.6.4 |
| PC5 | 10.32.6.5 |
| PC6 | 10.32.6.6 |
| PC7 | 10.32.6.7 |
| PC8 | 10.32.6.8 |
| PC9 | 10.32.6.9 |
| PC10 | 10.32.6.10 |
| PC11 | 10.32.6.11 |
| PC12 | 10.32.6.12 |
| PC13 | 10.32.6.13 |
| PC14 | 10.32.6.14 |
| PC15 | 10.32.6.15 |
| PC16 | 10.32.6.16 |
| PC17 | 10.32.6.17 |
| PC18 | 10.32.6.18 |
| PC19 | 10.32.6.19 |
| PC20 | 10.32.6.20 |
| PC21 | 10.32.6.21 |
| PC22 | 10.32.6.22 |
| PC23 | 10.32.6.23 |
| PC24 | 10.32.6.24 |

ESL :

| Device | IP address |
| --- | --- |
| PC1 | 10.32.7.1 |
| PC2 | 10.32.7.2 |
| PC3 | 10.32.7.3 |
| PC4 | 10.32.7.4 |
| PC5 | 10.32.7.5 |
| PC6 | 10.32.7.6 |
| PC7 | 10.32.7.7 |
| PC8 | 10.32.7.8 |
| PC9 | 10.32.7.9 |
| PC10 | 10.32.7.10 |
| PC11 | 10.32.7.11 |
| PC12 | 10.32.7.12 |
| PC13 | 10.32.7.13 |
| PC14 | 10.32.7.14 |
| PC15 | 10.32.7.15 |
| PC16 | 10.32.7.16 |
| PC17 | 10.32.7.17 |
| PC18 | 10.32.7.18 |
| PC19 | 10.32.7.19 |
| PC30 | 10.32.7.20 |
| PC21 | 10.32.7.21 |
| PC22 | 10.32.7.22 |
| PC23 | 10.32.7.23 |
| PC24 | 10.32.7.24 |
| PC25 | 10.32.7.25 |
| PC26 | 10.32.7.26 |
| PC27 | 10.32.7.27 |
| PC28 | 10.32.7.28 |
| PC29 | 10.32.7.29 |
| PC30 | 10.32.7.30 |
| PC31 | 10.32.7.31 |
| ACCESS POINT 1 | 10.32.7.32 |
| BigScreenTV | 10.32.7.33 |

CNL :

| Device | IP address |
| --- | --- |
| PC 1 | 10.32.8.1 |
| PC 2 | 10.32.8.2 |
| ACCESS POINT 1 | 10.32.8.3 |

MTR :

| Device | IP address |
| --- | --- |
| BigScreenTV | 10.32.9.1 |

MR :

| Device | IP address |
| --- | --- |
| ACCESS POINT 1 | 10.32.10.1 |

SR:

| Device | IP address |
| --- | --- |
| Server 1 | 10.32.0.1 |
| Server 2 | 10.32.0.2 |
| Server 3 | 10.32.0.3 |
| PC1 | 10.32.0.4 |
| PC2 | 10.32.0.5 |

DR:

| Device | IP address |
| --- | --- |
| PC1 | 10.32.0.6 |
| PC2 | 10.32.0.7 |

# Meeting Minute #1

| Date/Time |  | 20 DEC 2024 10.00 pm |  |
| --- | --- | --- | --- |
| Location |  | Online (Google Meet) |  |
| Agenda |  | 1. Understand the details about task 5. |  |
|  |  | 2. Discuss the way to divide the subnet. |  |
|  |  | 3. Complete task 5 together. |  |
| Meeting Host |  | Tan Zhi Ming |  |
| Attendance |  |  |  |
| Name |  | Time | Reason For Absence |
| Tan Zhi Ming |  | 2200 | - |
| Muhammad Naim Bin Abdullah |  | 2200 | - |
| Lee Yin Shen |  | 2200 | - |
| Ng Yu Hin |  | 2200 | - |
| Minutes |  |  |  |
| No. | Item Discussed | Details | Person-In-Charge |
| 1 | Discussion for task 5 | - Zhi Ming shared the information about task 5 and its rubric. | Naim |
|  |  | - All members read and investigate the task carefully. |  |
|  |  | - Solved the problem raised by group members together through discussion. |  |
| 2 | Suggestion for subnetwork division | - All members suggest their prefer subnetwork division - All members discuss and decide the one that best suits our design. | All members |
| 3 | IP Addressing Scheme Overview | - All members examine how IP addresses are currently assigned to each work area. | All members |
| 4 | Proceed to the parts of Task 5 | - All members work on google docs together to complete task 5. - At the same time, if any of the group members question and identify any issues or problems then we find ways to encounter problems together. | All members |
| 5 | Meeting Ended | - At 0130, the meeting ended after all the discussion and work was done. | All members |